# Toyota Celica TA22

(NSW RTA requirements as at 13/6/03 – May Not apply elsewhere)

Stock Tyres – 165 SR 13” (As per TA22 Celica Owners Manual – Scan Pictured below)

13” x 25.4mm = 330.2mm

165 x 82% = 135.3

330.2 + 135.3 + 135.3 = 600.8mm OD (601mm)

According to the RTA Vehicle Standards Information Sheet No. 9 “Guidelines for alternative wheels and tyres” The outside diameter of the wheel and tyre combination must be no more than 15mm over the largest diameter wheel and tyre combination specified for the vehicle and not more than 15mm below the smallest diameter wheel and tyre combination specified for the vehicle.

Therefore

Minimum OD is: 585.8 (586mm)

Maximum OD is: 615.8 (616mm)

Tyre Options. (May not list all legal options and not all sizes listed may be available)

13”

185/70 13” = 589mm

195/70 13” = 603mm

205/65 13” = 597mm

215/60 13” = 588mm

215/65 13” = 610mm

225/60 13” = 600mm

235/6013” = 612mm

14”

195/60 14” = 590mm

215/60 14” = 614mm

245/50 14” = 601mm

15”

195/55 15” = 596mm

205/50 15” = 586mm

205/55 15” = 607mm

215/50 15” = 596mm

225/50 15” = 606mm

235/45 15” = 592mm

235/50 15’ = 616mm

245/45 15” = 601mm

16”

195/50 16” = 601 mm

205/45 16’ = 591 mm

205/50 16” = 611 mm

215/45 16” = 600 mm

225/45 16” = 609 mm

235/40 16” = 594 mm